| INTENDED USE The ArmorWire TL-3 three-cable barrier (SGM33a) has been tested to NCHRP 350 criteria for longitudinal cable barriers. It can be used in either edge of road or median applications. The ArmorWire TL-3 system can use post spacing ranging from 1.0m to 9.0m in line with the installation deflection requirements (see interpolated deflection table below). Deflections marked with a star are from actual crash testing involving a 2000kg pickup truck impacting at 100kph at 25°. Post Spacing (m); 3.0; 4.0; 5.0; 6.0; 7.0; 8.0; 9.0. Deflection (m); 1.54*; 1.83; 2.12; 2.41; 2.69; 2.98; 3.27*. * Actual deflections. SGM33a is a Test Level 3 accepted system, by installing an additional cable to the top slot of the barrier post the system is converted to Test Level 4 (SGM33b). The remaining components are common for both TL-3 and TL-4 system. FEATURES AND SPECIFICATIONS - Exceptional vehicle control and behaviour - System with the lowest part count in the market - Same system can be used in roadside as well as median application - Tested and accepted to NCHRP 350 TL-3 (three wire system) - Easily upgradable to TL-4 system - Matching NCHRP 350 TL-3 end terminal available: Universal (ATE) - Minimal build-up in snow or sand environments - No sharp edges or exposed “hooks” on posts - Barrier width is only 97mm - Dynamic deflection for 2000 kg vehicle is 1.54m (100kph, 25°) - Length of Need is 8m (Terminal end LoN) - Posts are hot dip galvanized (powder coating is optional) - Concrete specified is 25 MPa (3625 psi) APPROVALS FHWA Acceptance Letter: B-222. CONTACT INFORMATION Armorflex International LTD www.armorflex.co.nz Telephone: +64 9 4269174 156 Foundry Rd Silverdale, Auckland 0932 New Zealand |
| --- |
| # ARMORWIRE CABLE BARRIER TL-3 |  |  |
| SGM33a |  |  |
| SHEET NO. | DATE |  |
| 2 of 2 | 07/10/2013 |  |