| INTENDED USE The ArmorWire TL-4 four-cable barrier (SGM33b) has been tested to NCHRP 350 criteria for longitudinal cable barriers. It can be used in either edge of road or median applications. Using a 3m post spacing the ArmorWire TL-4 system offers only 1.10m deflection when tested against the 8000 kg truck (80 kph and 15°). FEATURES AND SPECIFICATIONS - Exceptional vehicle control and behaviour - System with the lowest part count in the market - Same system can be used in roadside as well as median application - Tested and accepted to NCHRP 350 TL-4 (four wire system) & TL-3 (three wire system) - Matching NCHRP 350 TL-3 end terminal available: Universal (ATE) - Minimal build-up in snow or sand environments - No sharp edges or exposed “hooks” on posts - Barrier width is only 97mm - Dynamic deflection for 8000 kg vehicle is 1.10m (80 kph and 15°). - Length of Need is 8m (Terminal end LoN) - Posts are hot dip galvanized (powder coating is optional) - Concrete specified is 25 MPa (3625 psi) APPROVALS FHWA Acceptance Letter: B-222. CONTACT INFORMATION Armorflex International LTD www.armorflex.co.nz Telephone: +64 9 4269174 156 Foundry Rd Silverdale, Auckland 0932 New Zealand |  |  |
| --- | --- | --- |
| # ARMORWIRE CABLE BARRIER TL-4 |  |  |
| SGM33b |  |  |
| SHEET NO. | DATE |  |
| 2 of 2 | 07/10/2013 |  |